LETTER BALLOT
NB10-0302
Subj.:  Revision of S3.2 d)
Explanation:  The current text permits the repair firm to make repairs from non ASME Code material.  The proposed revision requires new parts to be made from Code material.

PM:  Shawn Malone

Background:  Typically, graphite parts are machined to shape or extruded, and then impregnated.  There are no standard specifications for impregnation of graphite, and no specifications for results of the impregnation.  Part UIG of Section VIII, Div. 1 requires extensive testing of impregnated graphite material to prove it meets Code requirements.  
The NBIC does not specifically require the repair firm to use ASME Code material for replacement parts.  The proposed revision addresses this issue.

Existing text:









Delete
[image: image1.emf]
Proposed Revision:

Replace with:

d) When ASME is the original code of construction, replacement parts subject to internal or external pressure shall be fabricated from certified material supplied by an organization having the appropriate ASME Certificate of Authorization.  The item shall be inspected and stamped as required by the applicable section of the ASME Code.  A completed ASME Manufacturer’s Partial Data Report shall be supplied by the impregnated graphite material manufacturer.

Rationale:  Part UIG of Section VIII, Div.1 controls the manufacture of impregnated graphite material, and requires the impregnated graphite to be stamped with the Code symbol.  The current words of S3.2 d) permits the repair firm to use parts, such as tubes, that are not Code material.  The proposed revision requires replacement parts to be fabricated from ASME stamped material.

